ROYAL INDUSTRIAL SUPPLIERS

GST NO.: 29AHZPU5620Q1ZZ

An ISO 9001:2015 Certified Company

QUALITY FASTENERS SPECIALIST

8310165941 / 9959457863	sales@royalfastenersindia.com        www.royalfastenersindia.com

TECHNICAL DATA SHEET

EYE BOLT

MATERIAL- STAINLESS STEEL

GRADE-SS304 (A2-70)

AS PER-DIN-580

CATALOGUE NO. R4G70

INSTALLATION GUIDE

- SELECT THE CORRECT SIZE AND TYPE OF EYE BOLT BASED ON LOAD REQUIREMENTS AND THREAD SIZE.

- ENSURE THE MOUNTING SURFACE OR MATERIAL IS CLEAN, FLAT, AND STRUCTURALLY STRONG ENOUGH TO HANDLE THE LOAD.

- VERIFY THAT THE THREADS OF THE EYE BOLT AND THE MATING HOLE ARE CLEAN AND FREE FROM RUST, DUST, OR DEBRIS.

- INSERT THE EYE BOLT INTO THE THREADED HOLE OR THROUGH-HOLE DEPENDING ON THE DESIGN.

- HAND-TIGHTEN THE EYE BOLT CLOCKWISE UNTIL IT CONTACTS THE MATING SURFACE.

- USE A SPANNER OR WRENCH ONLY ON THE SHANK FLATS (IF PROVIDED) AND NEVER ON THE EYE, TO AVOID DEFORMATION.

- IF USING A THROUGH-HOLE EYE BOLT, SECURE IT WITH A WASHER AND NUT ON THE OPPOSITE SIDE.

- APPLY THE RECOMMENDED TORQUE USING A TORQUE WRENCH ACCORDING TO LOAD AND SAFETY STANDARDS.

- ALIGN THE EYE PROPERLY IN THE DIRECTION OF THE LOAD TO PREVENT BENDING OR FAILURE.

- ENSURE THAT THE LOAD IS APPLIED IN LINE WITH THE PLANE OF THE EYE AND NEVER SIDEWAYS.

- CHECK THAT THE EYE BOLT IS FULLY SEATED, STRAIGHT, AND FREE FROM MOVEMENT.

- DO NOT EXCEED THE WORKING LOAD LIMIT SPECIFIED BY THE MANUFACTURER.

- INSPECT THE FINAL INSTALLATION TO CONFIRM A SAFE, STABLE, AND RELIABLE CONNECTION.

Note: Factor of safety is 6

Quote the PART NUMBER given below with your order/ inquiries:     	        All Dimensions in mm

| Nominal Size (d) | PART NUMBER | Stem Length | Eye-ID | Height | Collar Dia | Collar Height | Eye-OD | WLL Kgs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thread Size |  | L | d3 | h | d1 | e | d2 | T |
| M6x1.0 | R4G7006000 | 12 | 19.50 | 36.00 | 19.00 | 9.00 | 34.00 | 100.00 |
| M8x1.25 | R4G7008000 | 13 | 19.50 | 36.00 | 19.00 | 9.00 | 34.00 | 140.00 |
| M10x1.5 | R4G7010000 | 17 | 24.00 | 44.00 | 24.50 | 10.00 | 41.00 | 230.00 |
| M12x1.75 | R4G7012000 | 20 | 28.00 | 52.00 | 29.00 | 11.00 | 50.00 | 240.00 |
| M14x2.0 | R4G7014000 | 27 | 33.00 | 62.00 | 35.00 | 12.00 | 60.00 | 600.00 |
| M16x2.0 | R4G7016000 | 27 | 33.00 | 62.00 | 35.00 | 12.00 | 60.00 | 700.00 |
| M18x2.5 | R4G7018000 | 27 | 33.00 | 62.00 | 35.00 | 12.00 | 60.00 | 700.00 |
| M20x2.5 | R4G7020000 | 30 | 38.00 | 71.00 | 40.00 | 15.00 | 69.00 | 1200.00 |
| M24x3.0 | R4G7024000 | 36 | 48.00 | 90.00 | 50.00 | 19.00 | 86.00 | 1800.00 |
| M30x3.5 | R4G7030000 | 45 | 58.00 | 109.00 | 65.00 | 23.00 | 105.00 | 3600.00 |
| M36x4.0 | R4G7036000 | 54 | 72.00 | 126.00 | 70.00 | 28.00 | 120.00 | 3600.00 |

Sizes in BLUE are non-preferred. Availability is limited and MOQ is applied       Data is strictly informative